# How to Connect to the Arkin Lab Server with FileZilla

This guide explains how to set up FileZilla to connect to the Arkin Lab server using SFTP with OTP (One-Time Password) authentication.

## Step 1 – Open Site Manager

- Launch FileZilla.

- Go to File > Site Manager.

- Click New Site and give it a name (e.g., Arkin Server).

## Step 2 – Configure Connection

In the General tab, enter the following:

- Protocol: SFTP – SSH File Transfer Protocol

- Host: niya.qb3.berkeley.edu

- Logon Type: Interactive

- User: <yourusername>

## Step 3 – Connect

- Click Connect.

- FileZilla will first prompt you for your password.

- Then it will prompt you for your OTP code (just like in the terminal).

✅ Thanks to Vishant Gandhi for discovering this solution — setting Logon Type to Interactive is the key to making OTP work in FileZilla.

## Step 4 – Transfer Files

- Once connected, you can drag and drop files between your local machine and the server.

- Use the left panel for your local computer and the right panel for the server.

## Troubleshooting

- If FileZilla does not prompt for OTP, double-check that Logon Type is set to Interactive.

- Make sure you are connecting with SFTP (not FTP).

- If issues persist, test your login in the terminal using:

 sftp yourusername@niya.qb3.berkeley.edu

## ⚠️ Gotchas & Solutions

Problem: FileZilla repeatedly asks for your password or times out after a few file uploads (e.g., 'Connection timed out after 20 seconds of inactivity').

Cause: FileZilla opens multiple SFTP sessions when uploading multiple files at once. Since OTP authentication cannot be reused or entered automatically, new sessions fail once the first one expires.

Solution (thanks to Emma Rooholfada):

1. Go to Edit → Settings → Transfers

2. Set Maximum simultaneous transfers to 1

3. Go to Edit → Settings → Connection

4. Set Timeout in seconds to the maximum (9999)

This keeps a single persistent session alive and prevents repeated password/OTP prompts during uploads.